SLIDE 1

CADWorx Enhanced Specifications

Presented By:

Mark Bross

President of Kahuna Design

SLIDE 2

CADWorx Enhanced Specifications

What you can expect

CADWorx will have an enhanced pipe modeling system in the near future. With this comes a new specification tool that will allow for the new abilities in the enhanced modeling to exist. In this course you will get hands-on experience in using the new specification and catalog editor.

SLIDE 3

CADWorx Enhanced Specifications

- The NEW CADWorx Specification structure

- The Catalog/Specification Editor

- What’s my Catalog?

- How do I manage this?

- Can I customize it to fit my needs?

- Where do I save it for future projects?

- Can I bring the old stuff forward?

- 180 Bend is an elbow or any angle or radius not specific like old CadWorx. You can create 33 deg , LR SR etc. edit data table only.

SLIDE 4

CADWorx Enhanced Specifications

- End connections (End Types)

- What does that mean?

- Using one set of end connections for many components

- Setting up end types.

- Note: End types differ from End Prep ie. When putting in a piece of pipe the “End Prep” will be called out. Example is: a threaded connection will call out a threaded end for a pipe.

SLIDE 5

CADWorx Enhanced Specifications

- Copying and editing an existing spec

- Copy a new spec inside the spec editor

- Creating a spec from and existing SPC file

- Generating new specs to old ones?

SLIDE 6

CADWorx Enhanced Specifications

- Let’s get started

- Open the new Spec editor

- The new spec editor window areas

SLIDE 7

CADWorx Enhanced Specifications

- Create an Industry Catalog & take the default name of MyCatalog, or rename it if you like.

- This is used for your industry and can be changed to what you use as a Company or project catalog.

SLIDE 8

CADWorx Enhanced Specifications

- Drag & drop the categories and make a catalog

- Copy the categories into each new category of your new catalog

- Open the tables and look around

- Look at the properties for each part or table.

- Don’t forget to save your catalog.

SLIDE 9

CADWorx Enhanced Specifications

- Now that the new catalog is complete how do I use it?

- Making a New Specification

- Starting for scratch

- Using what is existing

- Customizing

- What Materials should I use?

- Selecting Materials

- Adding new Materials

Using a master list.

SLIDE 10

CADWorx Enhanced Specifications

- Using the same editor as the Catalog editor

- Start a new project (this is what they are calling a group of project specs these days.)

- Give it a name for your project ------( CAU-2011)

- Select your default catalog --- (MyCatalog.ids)

- Same as before you can change LD format etc.

- You can also give it a description if you want.

SLIDE 11

CADWorx Enhanced Specifications

SLIDE 12

CADWorx Enhanced Specifications

- Using the same editor as the Catalog editor we will now select Project   Add Spec to get the new specification dialog

- Give it a name  CAU-Spec1

- Fill out any Temp or Pressure Rating you want.

- Select the size table and fill in the Size range.

- Select the Thickness TableTHCK

- Select the Material Table  MAT

- Layer Name = Spec Name  or call it MYSpec

- Layer color = whatever you like ie. Yellow

- You can fill in the Created by or  Description if you like.

SLIDE 13

CADWorx Enhanced Specifications

SLIDE 14

CADWorx Enhanced Specifications

- Using the same editor as the project editor we will now right-click on the spec CAU-Spec1 to open the spec tab.

- Fill out the dialog box in the “Edit Component Window”

- Pick a “Group” in this case the Flanges

- Select the Type Table “Flange Weld Neck”

- Size range if you want different sizes.

- Select the Main Table  STD

- Select the Material Table  A105

- Fill in the Nominal Rating – Temperature & Pressure

- Layer Name = MYSpec

- You can fill in the Description if you like manually or with the fields available

SLIDE 15

CADWorx Enhanced Specifications

SLIDE 16

CADWorx Enhanced Specifications

- You can also add multiple components as well

- Right click in the spec tab window and select Add Multiple

- Select several components that you want in your spec

  - You can pick and choose

  - You can add more later

- If you need to you can select multiple endtypes as well by using the end type tables on the right side.

- You can also change schedule etc.

- Pick the “Add” button to populate the Spec Tab

- Then go through each selection and pick the appropriate data tables.

SLIDE 17

CADWorx Enhanced Specifications

SLIDE 18

CADWorx Enhanced Specifications   SLIDE 19

CADWorx Enhanced Specifications

SLIDE 19

CADWorx Enhanced Specifications

- The Branch TABLE

- This table will govern how branches are created in NG.

- Select the Branch table tab and start selecting the Branches to use

- Hold the control key and select the boxes as needed then Right Click and select the branch type.

SLIDE 20

CADWorx Enhanced Specifications

SLIDE 21

CADWorx Enhanced Specifications

- Now your spec is ready to use

- Start NG and try some pipe & flanges

DON’T FORGET TO SAVE YOUR WORK.

SLIDE 22

CADWorx Enhanced Specifications

THE BONUS ROUND!!!!

- Let’s import an existing spec

- Not just yet!!!!

- You need to create a Catalog of Legacy Data by Starting a new Catalog & importing the legacy data.

- Select “Import Legacy Data” from the Catalog pull down

SLIDE 23

CADWorx Enhanced Specifications

- Let’s import an existing spec

- Go to Project and select “Import Legacy Specification(.spc)”

SLIDE 24

CADWorx Enhanced Specifications

- Let’s import an existing spec

- Go to Project and select “Import Legacy Specification(.spc)”

SLIDE 25

CADWorx Enhanced Specifications